Abstract—Interference temperature has been proposed by the FCC as a metric for interference analysis.  The purpose of the metric is to demystify and remove the subjective context that has been the basis of interference analysis within the regulatory agencies. The development of an interference metric is critical if more intensive, dynamic use of the spectrum is desired.  There has been very little specified as to the origins, use, and challenges of an interference metric.  This paper provides an introductory analysis of the interference temperature metric.

Interference Temperature:

A Metric for Dynamic Spectrum Utilization

Paul J Kolodzy, PhD

Kolodzy Consulting

PO Box 1443

Centreville, VA 20120

703-266-9489

703-832-0254 (fax)

pkolodzy@kolodzy.com

# INTRODUCTION

The design of any RF transmission systems includes the analysis of potential interference created within the communications system (self-interference) and outside the system.  In fact, prior to deploying any new device, a detailed analysis is performed as to its impact to currently deployed devices. Engineering discipline uses a physical and mathematical description of interference to allow the engineers, given the interference constraints, to create an optimal design.  However, the current regulatory definition of interference is not precise.  It is a combination of physical realization and economic (and sometimes political) impacts.  The process used by the regulatory bodies assumes deployment geometry of the new devices and then performs a detailed analysis of the potential impact to incumbent devices and systems in terms of coverage loss, economic loss, etc.  This takes weeks, months, and sometimes years to reach a satisfactory conclusion.

The capacity for devices for waveform modification, frequency agility, and mobility creates challenges for the current regulatory process.  These new technologies can provide higher utilization of the RF spectrum and initiate new services for our highly mobile, digital society.  Without clear definition of interference, the ability to exploit these technologies could be curtailed.  A detailed analysis is practically impossible if all the possible combinations of waveforms and receivers are reviewed.   Additionally, currently deployed systems have difficulty automatically determining violations of their spectral interference rights.  Thus the current users of the RF spectrum have to expend resources to measure and analyze interfering signals and then subsequently locate and prosecute an source of those signals.  This process is even more difficult for short duration interference.  Therefore there is a need for both current spectral users as well as new entrants for a formal interference metric that can be used in real-time.

Interference metrics applications are not limited to regulatory analysis.  Mobile ad hoc networks (MANets) use interference metrics to define optimal power control mechanisms to improve throughput [1, 2].  Any mobile RF transmission system needs to define interference metrics in order to optimally use the RF spectrum.  Without it, the worst case geometry is used for interference analysis (i.e. the closest proximity).  Such geometry would results in stricter limits on transmission power.  Therefore, worst case analysis will limit transmission characteristics based on an unlikely interaction. This, in turn,  leaves much of our spectral resources fallow, unused.

The US Federal Communications Commission, in 2002, chartered an internal staff study, the Spectrum Policy Task Force, to investigate the future needs of RF spectrum and the limitations of current spectrum policies, as well as develop recommendations for enhancing current policies.  One recommendation was the use of an interference metric to enforce current spectrum access rights and create new opportunities for dynamic spectrum utilization. The development of an interference metric is critical if more intensive, dynamic use of the spectrum is desired [16, 17, 18, 19]. It proposed the interference temperature metric.  This paper will provide the motivation for the development of an interference metric, a description of interference temperature, example calculations, and technical challenges in applying the interference temperature metric.

# Driving Forces for Interference Metric

The central role of spectrum management is interference management.  The design and operation of RF equipment including communications and emitting non-communications devices are predicated upon preventing and/or mitigating electromagnetic interference. In today’s radio frequency environment, interference generally limits the useable range of communications signals.

Interference protection has always been a core responsibility of the communication regulatory bodies such as the Federal Communications Commission (FCC) in the U.S.  Section 303(f) of the Communications Act of 1934 as Amended directs the Commission to promulgate regulations it deems necessary to prevent interference between stations, as the public interest shall require [3].  This is still a critical aspect of the FCC operation.  The FCC’s strategic plan for the years 2003-2008 includes as a spectrum-related objective the “vigorous protection against harmful interference...”

Interference protection is addressed by regulatory rules on equipment through coordination or by negotiation.  Typically, regulatory agencies tailor their rules to the expected uses and technical characteristics of particular services at the time of their creation.  Most services have common elements aimed at interference protection such as limits on in-band power and out-of-band emissions.  Frequency coordination is a common means for interference management for many wireless and satellite services.  Negotiated interference agreements among affected parties also are permitted for numerous services.

The interference management methods employed varies from service to service. It is highly dependent upon when the service was initiated and the spectrum landscape at the time.  The rules are rarely updated unless a new service is employed that requires a change to an existing service.   Thus the interference management techniques can be considered static.

## Changing Landscape

The essentially static technique of interference management has been successful during the first 50 years of RF regulation.  Technology increased the available spectrum through new devices at a pace faster than new services were employed.  Therefore, static rules served the regulators well.  However, that is changing.

There has been a dramatic increase in overall demand for spectrum-based services and devices in recent years.  In particular, there has been a demand for mobile RF spectrum-based applications.  This increased demand has occurred  for traditional, licensed services and those offered through unlicensed devices.  This change is propelled by a host of factors: the economy has moved towards the communications-intensive service sector, the workforce is increasingly mobile, and consumers have been quick to embrace the convenience of wireless devices.

While technological advances are contributing to the increased diversity of spectrum-based consumer applications and, consequently, their use is resulting in more demand for spectrum, technological advances are also providing some potential answers to current spectrum policy challenges.  Recent and significant technological advances include the increased use of digital technologies and the development of smarter radios.

## Increased density and mobility

The developments of new devices and services have significantly changed the density of wireless transmitting devices.   In the early 1990s, transmitter densities of one per five people were common with Citizen Band radios, Amateur radios, etc.  Cellular telephony produced densities to that of one device per every two people.  Cordless phones, radio-controlled toys and other consumer products increase the density to approximately one per person. When wireless local area networks and wireless data services are included, the density is now at two per person (or greater).  There has been at least a 10x increase in density in 15 years.

Interference management has thus become more difficult because of the greater density, mobility and variability of radio frequency (RF) emitters.  Interference management becomes even more problematic, if and when users have been granted increased flexibility in their spectrum use. As a result, the complexity of predictive interference models has increased dramatically, and is expected to increase even more in the future.

# Origins of Interference Temperature

In 2002, the FCC created the Spectrum Policy Task Force to provide recommendations for future spectrum policy [4].  The SPTF, spurred by participants in public forums and by comments to a Public Notice, determined that there are rising concerns that current interference management paradigms will not adequately meet future spectrum demands.

Four basic challenges to spectrum management were outlined by the SPTF Interference Protection Working Group. First, many radio communications services have grown substantially in recent years.  A second challenge is the explosive consumer demand for RF devices.  According to NASA, “All the best allocation and assignment processes which maximize the use of the RF spectrum are to no avail if the RF environment becomes corrupted and interference becomes ‘harmful’ to radio services depending on that spectrum for fulfillment of mission goals.”[4] A third challenge developed as the technology of waveform flexibility in radios moved from a relatively small number of waveforms to widely varying signal architectures and modulation types for voice, video, data and interactive services. The fourth challenge encompasses rapidly advancing technology; its use will continue to change the interference landscape.  For example, due to advances in digital signal processing and antenna technology, communications systems and devices are becoming more tolerant of interference through their ability to sense and adapt to the RF environment.

Additionally, the increased ability of new technologies to monitor their local RF environment and operate more dynamically than traditional technologies can provide new mechanisms for interference management.  The predictive models used by government regulators and network operators can be updated, and perhaps eventually replaced, by techniques that take into account and assess actual, rather than predicted, interference.

## SPTF Proposal

The SPTF proposed one interference metric, the inter-ference temperature.”  The interference temperature is a measure of the RF power available at a receiving antenna to be delivered to a receiver – power generated by other emitters and noise sources.   More specifically, it is the temperature equivalent of the RF power available at a receiving antenna per unit bandwidth, measured in units of ºKelvin.  The FCC SPTF used the terms “interference temperature” and “antenna temperature” synonymously.

Use of the interference temperature concept would be more amenable to an RF interference environment having the properties of additive Gaussian white noise.  For such signals, the received power at the output terminals of the antenna could be calculated as the product of the interference temperature and the bandwidth:

P=kTB

where P is the power, k is Boltzmann’s Constant (1.38x10-23 W Hz-1K-1), T is the interference temperature (K•Hz-1).

As illustrated in Figure 1, interference temperature measurements could be taken at receiver locations throughout the service areas of protected communications systems, thus estimating the real-time conditions of the RF environment.

Like other representations of radio signals, instantaneous values of interference temperature would vary with time and, thus, would need to be treated statistically.  One system concept calls for continuous monitoring of particular frequency bands, measuring and recording interference temperature values and computing appropriate aggregate value(s).  These real-time values could govern the operation of nearby RF emitters.  Measurement devices could be designed with the option to include or exclude the on-channel energy contributions of particular signals with known characteristics; e.g., the emissions of users in geographic areas and bands where spectrum is assigned to licensees for exclusive use.

Figure 1 Interference Management Differentiation between the Transmitter and the Receiver (from [4[).

The Commission could use the interference temperature metric to set maximum acceptable levels of interference, thus establishing a “worst case” environment in which a receiver would operate.  Interference temperature thresholds could thus be used, where appropriate, to define interference protection rights.  Threshold levels could be set for different bands, geographic regions or services.  These levels could serve as benchmarks to guide engineering tradeoffs for radio equipment and system designers.

# Examples of Interference Temperature Calculations

It is useful to compute the interference temperature,IT, for a few sample cases.  The first case will be for a Class B Part 15 device operating at 800 MHz and 1200 MHz.  The field strengths limits measured at 3 meters permitted by FCC rules are 46 dB and 54 dB (uV/m) respectively.  This can be converted to power flux density by the relationship

S=E2377Ω,

which is computed to be -99.7 dB(W/m2) and -91.7 dB (W/m2) for 800 MHz and 1200 MHz.  The translation from power flux density to interference temperatures assumes unit bandwidth

IT=Sk,

where k is Boltzman’s constant.  The computed interference temperatures are 128.9 dB Kelvin Hz-1 and 136.9 dB Kelvin Hz-1 for the 800 MHz and 1200 MHz device respectively.

Another interesting value is the interference temperature for a moderately sophisticated digital receiver.  A typical digital receiver for 25 KHz signals has a noise floor of -105 dBm.  Assuming a dipole antenna operating at 800 MHz, the effective antenna areas, Aeff, is computed using

Aeff=λ24π,

which is 0.011 sq meters or -19.5 dB sq meters.  Thus the power flux density would be -115.5 dB(W/m2) and the corresponding interference temperature of  113.1 dB Kelvin Hz-1.  That is, the sensitivity of the digital receiver is such for a detection threshold of 113.1 db K Hz-1.

The values of the interference temperature are more relevant when put into context of actual RF environments. It is also useful to compare to noise temperature values that are used in radio astronomy.  Average sky noise temperatures are approximately 1000 Kelvin (30 dB K) at 100 MHz and 10 - 40 Kelvin (10-16 dB K) at 1-10 GHz [6].  For example, a 1 GHz BW ultra-wideband (UWB) signal operating at the peak Part 15 Class B values above 1000 MHz (e.g. 1200 MHz) could produce an interference temperature of 46 dB K (=136.9-90.8).  The digital receiver described above would need to be integrating across 50MHz to be able to register the signal!  Narrower bandwidths and various receiver designs will generate a wide range of results.

The previous example used the maximum power flux density to compute the expected interference temperature from a single device at a distance of 3 m.  Another example would be to use a 100 mW transmitter operating at 2.4 GHz.  Therefore the power flux density would be

S=Pt4πR2	   = 	-30.5 dB (W/m2) at 3 meters; and

=    -50 - -70 dB (W/m2) at 30 meters.

The range of values for the 30 meter case is to capture the potential variations expected from an R2 and R4 propagation environment.  Given the bandwidth of the signal of 20 MHz, the interference temperature would be 135 dB Kelvin Hz-1 at 3 m to 95 – 115 dB Kelvin Hz-1 at 30 m.

# Community Reaction

There has been varied reaction from of the different communities to the development a common interference metric and to the proposed interference temperature.  The policy community has had  both positive and negative reactions to the notion of interference metrics.  A great deal of the negative reaction can be linked to the FCC NOI [7].  The NOI linked spectrum management to the interference metric and thus has provided an impression that the acceptance of the interference temperature is also an  acceptance of spectrum sharing and more unlicensed use of the spectrum [8,9],   Many incumbent spectrum users are concerned with the possibility of more spectrum sharing.  However, the “Open Spectrum” community has been very supportive [10].  Many in the community promoting the use of unlicensed devices have indicated interest tempered by the lack of technical specificity [11, 12].  Finally, some in the engineering community are concerned with the potential of a general increase in interference due to changing the interference rules [13].

The scientific community has readily embraced the notion of developing interference metrics [14]. As with some in the unlicensed community, there is a concern with the lack of definition and scientific rigor [15].  The National Science Foundation through the NeTS program has been investigating both metrics and means to measure interference and spectrum occupancy.  The IEEE ElectroMagnetic Compatibility (EMC) Society has a renewed interest in interference metrics, especially for dynamic spectrum access.  The IEEE EMS and IEEE Communication Society have jointly initiated the 1900 Standard Series on Next Generation Radio and Spectrum Management to address the standards for dynamic spectrum.  In particular, the IEEE 1900.2 working group is developing “recommended practices for interference and coexistence analysis”.  Thus, the research community has begun to address the technological challenges in measuring and predicting interference.

# Challenges

Two primary technology challenges face the use of the interference temperature as an overall metric: (1) the determination of the “background” interference environment as a function of spatial location and frequency; and (2) the in situ measurement of the interference temperature to determine optimal radio transmission parameters.  The research and development community, such as Shared Spectrum Corporation (SSC) and Stevens Institute of Technology (WiNSeC), have been actively developing measurement equipment and techniques with which to estimate the background interference environment.  The in situ measurements for real time RF monitoring have yet to be actively addressed.

Figure 3 Hoboken NJ Measurements in the 800 MHz Band by WiNSeC at Stevens Institute.

Figure 2 NYC Measurements from 138-174 MHz conducted by Shared Spectrum Corporation

Examples of the RF measurements are provided in Figures 2 and 3.  Figure 2 provides the power during an 18 hour measurement interval as well as time-frequency intervals that exceeded a 90 dBm occupancy threshold.  The measurements shown in Figures 2 and 3, as with many spectrum measurements, were made at single locations.  These measurements, given the antenna characteristics, can be converted to power flux density and thus to an interference temperature.  However, an interference temperature at one point cannot provide an interference map over an entire area.  This is especially true in the presence of Rayleigh fading.

The deployment scenario for in situ RF monitoring can either be centralized, distributed, or a hybrid of the two. The centralized technique uses measurements from many fix and mobile sites and integrate the data to create an overall power flux density map across a large area. The distributed technique uses measurements from either single site or limited local sites to create a localized power flux density map. The centralized method will be more accurate; however it will require substantially more communications resources.  The distributed method will be less accurate but easier to deploy.  In either case, the spatial sampling will be insufficient to accurately map multipath; models will need to be used to fill the spatial gaps.  Thus an engineering tradeoff must be made between the precision of the power flux density measurements, the spatial sampling of the RF environment, and the fidelity/robustness of the propagation models.

## Distributed Measurements

The fidelity requirements on the understanding of the RF environment are dependent upon the application.  The fidelity requirement for interference metrics is to provide the most conservative estimate to prevent undesired interference.  It will be assumed that digital data will be transmitted.  The expected use of the interference temperature is to 1) determine that the interference levels are within the limits set by the regulatory body; and 2) to determine that the impact of an RF emission is such that the interference levels remain under those limits.  The second use will be the focus of the remainder of this section since it already includes determining the interference levels that is called out in the first use.

The impact of a new RF transmission on the power flux density is a combination of 1) the power flux density already present, 2) the additional power due to the transmitter, and 3) the combining mechanism (coherent, incoherent).

The maximum power added to the environment from the transmitter can be computed by assuming an EIRP emission with R2 propagation.  Computing the power flux density using spherical propagation provides an upper bound and thus the maximum interference protection.  It would then follow to use the assumption of coherent power addition in order to compute an upper bound.  However the digital transmission assumption allows the use of incoherent addition (average power) while still maintaining a conservative estimate on interference..

The determination of the RF map (power flux density, frequency, space) will be the focus of the remainder of this section.  This will require accurate sampling of the interference temperature in space and estimating the interference temperature in regions throughout the sampled space.

Although a receiver can obtain a fairly accurate estimate of the interference temperature at its antenna, it is important to note that the measurements must be made to reflect only large-scale signal fading from the sources of the interference.  Due to potential of small-scale fading (as shown in Figure 4), multiple samples in space around the location will be needed to process out any small-scale fades.  The optimal sampling criteria, not discussed in this paper, depends on transmitter/receiver geometry, terrain feature (natural and man-made), and transmission frequency.

The second problem is to extrapolate the interference temperature measurement to any arbitrary point.  If the location of the interferer is known, then the extrapolated value can be computed using the known measurement and either a R2 or R4 ratio.  Since the estimate should be biased high (for the greatest protection), then the R4 ratio (or greater) would be used if computing for a location that is closer to the interferer and R2 for a location that is further from the interferer. It should be noted that this calculation is valid for either a single interferer or multiple interferers with known relative EIRP.

Figure 4 Large Scale and Small Scale Fading

The importance of knowing the position of the interferer is predicated upon the extrapolation distance compared to the overall distance to the interferer.  Figure 5 graphically depicts the problem for a single contributor.  If the transmitter location is known, then a single estimate of the large-scale fading is sufficient to determine the relative contributions of distant interferers.  Given the assumption that the error in the measured power flux density is small, then the error in the estimated power flux density at a range offset ∆R from the measurement position is given bounded by

dSS≤dRR for ΔRR≈1; and

dSS≈0 for ΔRR<<1.

That is, the error in the estimated power flux density, or equivalently interference temperature, has an upper bound of the error in the estimated range between the interfering source and the transmitter.  That is shown in Figure 5.

The more difficult case for estimating the interference temperature is when the positions of the interferers are unknown.  Therefore, multiple measurements at varying positions are needed to reduce the error in the estimate of the slope of the power flux density.

Figure 5 Inteference Estimation and the Impact of Geometry.  Close Interferers have larger variability of signal power and thus the interference temperature estimate away from the receiver is more challenging.

This is an area of research. Such an estimate will require a number of measurements to be combined. Figure 6 is an example of four simultaneous measurements within a 5 meter radius within an office building in the 2450-2485 MHz band.  Producing useful distributed measurements will have challenges in calibration, sensitivity, and synchronization.

# Conclusion

There is little doubt in the scientific, policy, and user communities as to the value of a common interference metric [20].  The ability to quantify the RF environment has benefits from insuring the interference rights of spectrum users, to the capacity to efficiently adapt to environment dynamics, and the potential to provide new opportunities to use the spectrum more intensively.  The interference temperature is one such metric.  The challenges are still areas of research and debate but the advantages dictate that their solutions be actively pursued.  The research community should vigorously look into the technology to measure the interference, the interpretation of those measurements, and the methodology to set limits on transmission based upon the interpretation.  There is the potential to open the RF spectrum to new and innovative management techniques for more efficient and greater use of this limited and valuable resource.

Acknowledgment

Figure 6 Simultaneous time-frequency measurements within an office building in the upper portion of the WiFi band.  Separation distances of 5 meters or less.

The author would like to acknowledge the engineers, scientists, and attorneys at the FCC who contributed in the development of the interference temperature concept.  In particular, Keith Larson who originated many of the ideas and Michael Marcus who was instrumental in the analysis are gratefully acknowledged.

References

- D.M.Blough, M.Leoncini, G.Resta, P.Santi,“The k-Neighbors Approach to Interference Bounded and Symmetric Topology Control in Ad Hoc Networks”, IEEE Trans. on Mobile Computing (to appear). Paper (.pdf), 2005.

- T Johansson & L Carr-Motyckova, “Reducing Interference in Ad hoc Networks through Topology Control”,  The Third ACM/SIGMOBILE International Workshop on Foundation of Mobile Computing , 2005  http://gridnet.nec-labs.com/dialm/pdf/p17-johansson.pdf

- United States Code, Title 47, Section 303, paragraph (f), also http://www4.law.cornell.edu/uscode/ .

- “FCC Spectrum Policy Task Force Report”, www.fcc.gov/sptf , November 2002.

- Wolfram Research at http://scienceworld.wolfram.com/ physics/AntennaTemperature.html

- Sklar, Digital Communications, Prentice Hall, 2001, Chapter 5.

- Federal Communications Commission, Notice of Inquiry and Notice of Proposed Rulemaking: In the matter of Establishment of an Interference Temperature Metric to Quantify and Manage Interference and to Expand Available Unlicensed Operation in Certain Fixed, Mobile and Satellite Frequency Bands, ET Docket No. 03-237; November 28, 2003

- J Bravo, H Kopeck, R Tai, J Velasco, D Wong, “FCC Spectrum Policy: Reforming Policy to Incorporate Emerging Technologies”, UCLA School of Public Policy & Social Research March 25, 2004.

- “Industry urges Martin to reject Interference temp”, RCRNews, August 29,2005.

- Ikeda, Nobuo and Lixin Ye, “Spectrum Buyouts: A Mechanism to Open Spectrum,” RIETI Discussion Paper, December 2003;  http://www.rieti.go.jp/jp/publications/summary/02030001.html.

- W. Lehr, “Economic Case for Dedicated Unlicensed Spectrum Below 3GHz, New America Foundation Spectrum Series, Working Paper #9, July 2004.

- W Horne, “Adaptive Spectrum Access: Using the Full Spectrum Space”, MITRE Corp., TPRC Proceedings, 2003.

- J Ely, T Shaver, G Fuller, “Ultrawideband Electromagnetic Interference to Aircraft Radios”, NASA/TM-2002-211949; January 2005.

- D Febus, “An Interference Temperature Model For Improving Spectrum Access”, IREAN Research Workshop; Feb 2005, http://www.vsgc.odu.edu/conf05/Papers/Paper%20-%20D.%20Maldonado.pdf.

- F Berggren, et al, “Dynamic Spectrum Access - Phase 1: Scenarios and research challenges; TRITA-S3-RST-0407; ISSN 1400-9137; ISRN KTH/RST/R--04/07—SE, 2004, http://www.wireless.kth.se/projects/DSA/DSA_report_phase1.pdf.

- D. Cabric, S.M. Mishra, and R.W. Brodersen, “Implementation Issues in Specrum Sensing for Cognitive Radios”, Conference Record of the Thirty-Eighth Asilomar Conference on Signals, Systems and Computers, 2004.

- V.D. Chakravarthy, A.K. Shaw, M.A. Temple, and J.P Stephens, “Cognitive Radio – an Adaptive Waveform with Spectral Sharing Capability”, IEEE Wireless Communications and Networking Conference, 2005.

- P. Kolodzy “Dynamic Spectrum Policies: Promises and Challenges”, CommLaw Conspectus, 2004.

- J Walko, “Wireless and the spectrum - Cognitive radio - Wireless spectrum is a scarce, valuable resource that's unused for much of the time. can cognitive radio put an end to this apparent paradox?”, IEE Review, May 2005.

- R. Paul Margie, “Efficiency, Predictability, And The Need For An Improved Interference Standard At The FCC”, TPRC 2005.

Paul J Kolodzy  Dr.Kolodzy has 20 years of experience in technology development for advanced communications, networking, electronic warfare, and spectrum policy for government, commercial, and academic clients.  He is currently a Technology Consultant in Advanced Wireless and Networking Technology. Dr. Kolodzy has been:  Director of the Center for Wireless Network Security (WiNSeC) at Stevens Institute of Technology; the Senior Spectrum Policy Advisor at the Federal Communications Commission (FCC) and Director of Spectrum Policy Task Force charged with developing the next generation spectrum policy; Program Manager at the Defense Advanced Projects Agency (DARPA) in the Advanced Technology Office managing R&D for communications programs to develop generation-after-next capabilities; Director of Signal Processing and Strategic Initiatives at Sanders, A Lockheed Martin Company; and a Group Leader/Staff Member at MIT Lincoln Laboratory.   He received his PhD and MS in Chemical Engineering from Case Western Reserve University and his BS in Chemical Engineering from Purdue University.

See Federal Communications Commission Strategic Plan FY 2003 – FY 2008, available at  http://www.fcc.gov/omd/strategicplan2003-2008.pfd.

The spectrum landscape denotes the available devices and their spectral utilization.

This density discounts the array of low-power Part 15 devices such as garage door openers and baby monitors.

The related term “noise temperature” is used by radio astronomers as a measure of the intensity of radiations from space.

The idea of an interference temperature as a measure of the antenna “noise” power in a particular band and location is well established.  See [5]..

It may be technically complex to estimate the interference temperature excluding the licensed desired users.

Acceptable “interference temperature” limits could, in effect, provide implicit receiver standards, because equipment manufacturers would have the option of designing receivers to operate in “worst case” RF environments.

Of course, a higher antenna gain that is aligned with the UWB transmitter can reduce the required integration bandwidth.

See www.sharedspectrum.com

See www.stevens.edu/winsec